KisanBot — AI Farmer Assistant

Name:

Thameemul Ansari U(22MIS1140)

Shirish S(22MIS1121)

# 1. Project Overview

KisanBot is an offline-capable AI-powered voice assistant designed specifically for Indian farmers. It runs entirely on a Raspberry Pi 4B — a $35 single-board computer — requiring no internet connection for its core functionality. Farmers interact with it by pressing a physical button, speaking a question in any Indian language, and receiving a spoken answer in return.

The project targets the 600 million Indians who depend on agriculture, many of whom live in areas with poor internet connectivity, speak regional languages, and are not tech-literate. KisanBot solves all of these problems simultaneously.

# 2. Hardware Used

| Component | Model / Spec | Purpose |
| --- | --- | --- |
| Raspberry Pi 4B | 8GB RAM, 64-bit | Main compute unit — runs all AI models |
| MicroSD Card | 32GB+ | Stores OS, models, and farming database |
| 16x2 LCD Display | I2C module (address 0x27) | Shows status messages and AI responses |
| Push Button | Tactile switch on breadboard | Push-to-talk trigger for voice input |
| USB Headset | Dell WH3024 | Microphone input for voice recording |
| 3.5mm Headphones | Any wired headphones | Audio output for spoken AI responses |
| Breadboard + Jumper Wires | Standard | Circuit connections for button and LCD |
| Powerbank / Adapter | 5V/3A USB-C | Power supply for the Pi |

# 3. Software & Technologies

## 3.1 Operating System

Raspberry Pi OS (Bullseye, 64-bit) — A Debian-based Linux distribution optimized for Raspberry Pi hardware. The 64-bit version is required for running modern AI models and libraries.

## 3.2 AI Language Model — Phi3 Mini (via Ollama)

Phi3 Mini is a small but powerful large language model developed by Microsoft, run locally using Ollama. It serves as the brain of KisanBot — understanding farmer questions and generating practical agricultural advice.

- Model size: ~2GB (quantized Q4 format)

- Runs entirely on CPU — no GPU required

- Response time: 30-90 seconds on Pi 4B

- Completely offline — no API calls needed

## 3.3 Speech-to-Text — OpenAI Whisper

Whisper is an open-source automatic speech recognition (ASR) model developed by OpenAI. It converts the farmer's spoken voice into text that the AI model can process.

- Model used: Whisper Tiny (fastest, lowest RAM)

- Supports 99 languages including Hindi, Tamil, Telugu, Marathi, Bengali, Punjabi

- Automatically detects the language — no configuration needed

- Runs fully offline on the Pi's CPU

## 3.4 Text-to-Speech — espeak

espeak is a lightweight open-source speech synthesizer that converts the AI's text response back into spoken audio. It supports Indian languages and runs with minimal RAM usage.

- Supports Hindi, Tamil, Telugu and other Indian languages

- Extremely lightweight — runs instantly on Pi

- Audio output via 3.5mm headphone jack

## 3.5 Vector Database — TF-IDF with hnswlib

A vector database stores farming knowledge as mathematical representations (vectors) that capture semantic meaning. When a farmer asks a question, the system finds the most relevant knowledge chunks by comparing meaning — not just keywords.

- Technology: TF-IDF vectorization + cosine similarity

- Library: scikit-learn + hnswlib

- Contains 10+ Indian farming knowledge entries (expandable)

- Retrieves top 2 most relevant knowledge chunks per query

- Runs completely offline with zero latency

## 3.6 RAG Architecture (Retrieval Augmented Generation)

RAG is an AI technique that combines a knowledge database with a language model. Instead of relying solely on the AI model's training data, relevant farming knowledge is retrieved first and injected into the AI's prompt — making answers more accurate and India-specific.

## 3.7 16x2 LCD Display — RPLCD Library

The LCD display shows real-time status updates throughout the conversation — 'Listening...', 'Thinking...', 'Speaking...' — giving the farmer visual feedback even without a screen.

## 3.8 GPIO Button — RPi.GPIO

A physical push button connected to GPIO Pin 17 acts as the push-to-talk trigger. When held down, it starts recording the farmer's voice. This makes the device accessible to non-tech-literate users.

## 3.9 Weather & Market APIs (Online Mode)

When internet is available, KisanBot automatically fetches live data to enrich its responses:

- OpenWeatherMap API — current weather conditions for the farmer's region

- data.gov.in Agmarknet API — live mandi (market) prices for crops by state

# 4. System Architecture

The complete pipeline from button press to spoken response:

| Step | Component | What Happens |
| --- | --- | --- |
| 1 | GPIO Button (RPi.GPIO) | Farmer presses button → Pi starts recording audio |
| 2 | USB Headset Mic | Voice is captured as 48kHz WAV audio file |
| 3 | Whisper (STT) | Audio converted to text, language auto-detected |
| 4 | Vector DB (TF-IDF) | Relevant farming knowledge retrieved for context |
| 5 | APIs (if online) | Live weather and mandi prices fetched |
| 6 | Phi3 Mini (LLM) | AI generates practical farming advice using context |
| 7 | espeak (TTS) | Text response converted to speech audio |
| 8 | 3.5mm Speaker | Farmer hears the answer spoken aloud |
| 9 | 16x2 LCD | Status and response summary shown on display |

# 5. Key Features

- Fully offline operation — works in villages with zero internet

- Multilingual — supports all major Indian languages via Whisper

- Voice-first interface — no reading or typing required

- RAG architecture — India-specific farming knowledge base

- Live data integration — weather and mandi prices when online

- Edge AI — entire AI stack runs on a $35 computer

- Push-to-talk button — simple enough for any farmer to use

- LCD feedback — real-time status display

# 6. Project Files

| File | Purpose |
| --- | --- |
| main.py | Main application — ties all components together |
| vectordb.py | Vector database setup and query functions |
| farming_data.py | Indian farming knowledge dataset |
| apis.py | Weather and mandi price API integrations |

# 7. How to Run

- Power on Raspberry Pi 4B

- Connect to WiFi or phone hotspot

- SSH into Pi: ssh pi@172.20.10.2

- Navigate: cd farmer_assistant && source venv/bin/activate

- Run: python3 main.py

- Wait for 'Farmer Assistant Ready' on LCD

- Press button and speak your farming question!

# 8. Real-World Impact

KisanBot directly addresses three critical barriers facing Indian farmers:

- Connectivity barrier — works completely offline in remote villages

- Language barrier — supports all major Indian languages natively

- Literacy barrier — voice interface requires no reading or typing

With over 600 million Indians depending on agriculture, and most of them lacking access to timely, accurate farming advice in their native language, KisanBot represents a practical, affordable solution that can be deployed at scale using low-cost hardware.

# 9. Implementation ScreenShot